INQUIRY WORKBOOK

# Introduction to Drone Remote Sensing for Environmental Science

Student Workbook and Capstone Data Packet

| AUDIENCE AP Environmental Science and introductory undergraduate environmental science | LENGTH 10 instructional days | RESOURCE Student-facing activities |
| --- | --- | --- |

Driving question: How can drone-based remote sensing help environmental scientists observe, measure, and manage environmental change?

# How to use this workbook

This unit treats remote sensing as a measurement and reasoning process. You will repeatedly separate what a sensor records from what an environmental scientist infers. You do not need to operate a drone or use specialized software.

Evidence habit Use the sequence Observe -> Quantify -> Interpret -> Question uncertainty -> Recommend a next step.

# Unit vocabulary and quick reference

| Term | Working definition |
| --- | --- |
| Remote sensing | Obtaining information about an object or area without direct physical contact, using recorded electromagnetic energy or ranging. |
| sUAS | Small uncrewed aircraft system: aircraft plus control, communication, payload, and supporting components. |
| Passive sensor | Records naturally reflected or emitted energy; examples include RGB, multispectral, and thermal cameras. |
| Active sensor | Emits energy and measures its return; lidar is a common example. |
| Reflectance | Proportion of incoming energy reflected by a surface at a wavelength. |
| Spectral band | A defined wavelength interval recorded by a sensor. |
| RGB | Red, green, and blue visible-light bands used for natural-color imagery. |
| Multispectral | Several discrete bands, often including wavelengths beyond human vision. |
| Thermal infrared | Emitted energy associated with surface temperature; apparent temperature requires careful calibration and context. |
| Lidar | Active laser ranging used to estimate distance and three-dimensional structure. |
| Spatial resolution | Ground dimensions represented by a pixel, often called ground sampling distance for drone imagery. |
| Spectral resolution | How narrowly and how many wavelength regions a sensor distinguishes. |
| Temporal resolution | How often the same location can be observed. |
| Radiometric resolution | Sensitivity to differences in recorded energy intensity. |
| NDVI | Normalized Difference Vegetation Index: (NIR - Red) / (NIR + Red). |
| Orthomosaic | A georeferenced, geometrically corrected mosaic made from overlapping images. |
| Photogrammetry | Deriving measurements and 3D information from overlapping photographs. |
| Point cloud | A collection of three-dimensional points representing surfaces or objects. |
| DSM / DTM | A digital surface model includes surface features; a digital terrain model aims to represent bare ground. |
| Ground control point | A clearly identifiable point with surveyed coordinates used to improve or evaluate positioning. |
| Validation | Independent observations used to test whether a remote interpretation is accurate. |
| Uncertainty | A quantified or described limit on confidence arising from measurement, sampling, processing, or interpretation. |

# Sensor selection reference

| Sensor | Directly records | Useful for | Watch for |
| --- | --- | --- | --- |
| RGB | Visible reflected light | Land cover, erosion, visible damage, mapping | Lighting, shadows, color balance |
| Multispectral | Selected reflected bands | Vegetation indices, water/soil contrasts | Calibration, species/season effects |
| Thermal | Emitted infrared energy | Surface-temperature patterns, leaks, water stress | Time of day, emissivity, weather |
| Lidar | Laser travel time/range | Elevation and vegetation structure | Cost, density, classification |

## NDVI calculation frame

NDVI = (NIR - Red) / (NIR + Red)

Check that NIR and Red use comparable reflectance units.

Calculate the numerator and denominator separately.

Values commonly range from -1 to +1.

Interpret patterns in ecological context; NDVI is not a diagnosis.

# Day 1: Seeing the Environment from Above

Essential question What can an aerial view reveal that a ground-level view may miss?

Name: ______________________________    Date: __________________

## Activity 1A - Notice, then interpret

| Neutral observation | Possible interpretation | Evidence needed to test it |
| --- | --- | --- |

Which pattern is easiest to see from above? Why?

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Exit ticket: Name one phenomenon suited to remote observation, one advantage, and one needed ground check.

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

# Day 2: A Drone as a Scientific Measurement System

Essential question How does a drone become more than a flying camera?

Name: ______________________________    Date: __________________

## Activity 2A - Build a defensible mission

Scenario: A manager suspects that an invasive wetland plant has expanded across a 40-hectare preserve. Complete the workflow.

Environmental decision and research question

________________________________________________________________________________

Sensor and the physical measurement it records

________________________________________________________________________________

Flight/coverage information needed

________________________________________________________________________________

Ground observations needed

________________________________________________________________________________

Processing output

________________________________________________________________________________

Quality-control checks

________________________________________________________________________________

Evidence that would support or challenge the interpretation

________________________________________________________________________________

# Day 3: Sensors and the Electromagnetic Spectrum

Essential question Which sensor can answer which environmental question?

Name: ______________________________    Date: __________________

## Activity 3A - Choose the sensor

| Mission | Best starting sensor | Measurement-based justification |
| --- | --- | --- |
| Detect crop stress before strong visible symptoms |  |  |
| Locate unusually warm water entering a stream |  |  |
| Map detailed canopy height and terrain structure |  |  |
| Document visible shoreline change after a storm |  |  |

Select one mission. What would the sensor record directly, and what would you infer?

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

# Day 4: Resolution and Scale

Essential question How much detail is enough to answer the question?

Name: ______________________________    Date: __________________

## Activity 4A - Resolution laboratory

| Feature | Approx. width | 2 cm pixels | 10 cm | 1 m | 10 m | 30 m |
| --- | --- | --- | --- | --- | --- | --- |
| Single leaf | 8 cm |  |  |  |  |  |
| Young tree crown | 2 m |  |  |  |  |  |
| Wetland patch | 25 m |  |  |  |  |  |
| Forest stand | 300 m |  |  |  |  |  |

For each cell, mark D (likely detectable), U (uncertain), or N (not reliably detectable). Then explain two choices.

Why might the finest resolution not be the best operational choice?

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Exit ticket: Explain the 30 m forest / individual-tree contrast.

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

# Day 5: Multispectral Data and NDVI

Essential question How can red and near-infrared reflectance reveal vegetation patterns?

Name: ______________________________    Date: __________________

## Activity 5A - Calculate and interpret NDVI

| Point | Red | NIR | NDVI | Careful interpretation |
| --- | --- | --- | --- | --- |
| P1 | 0.2 | 0.6 |  |  |
| P2 | 0.3 | 0.45 |  |  |
| P3 | 0.12 | 0.1 |  |  |
| P4 | 0.18 | 0.54 |  |  |
| P5 | 0.28 | 0.36 |  |  |

Write one defensible interpretation and one overclaim for P1.

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

List two non-health factors that can influence NDVI.

________________________________________________________________________________

________________________________________________________________________________

# Day 6: From Photographs to Maps

Essential question Why are overlapping photographs not yet a reliable map?

Name: ______________________________    Date: __________________

## Activity 6A - Product and process match

Number these steps from 1 to 7: ____ collect overlapping photographs  ____ environmental interpretation  ____ identify research question  ____ feature matching and camera estimation  ____ build point cloud  ____ produce orthomosaic/model  ____ verify position and quality

| Product | What it represents | One limitation |
| --- | --- | --- |
| Individual photograph |  |  |
| Orthomosaic |  |  |
| Point cloud |  |  |
| Digital surface model |  |  |
| Digital terrain model |  |  |

Why can a surface model differ from bare-earth elevation?

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

# Day 7: Environmental Applications Case-Study Stations

Essential question How does the environmental question shape the sensing strategy?

Name: ______________________________    Date: __________________

## Precision agriculture

A grower sees uneven crop vigor before harvest.

| Question | Team response |
| --- | --- |
| Sensor and why |  |
| Fit-for-purpose resolution |  |
| Useful remote product |  |
| Ground data |  |
| Important limitation |  |

## Wetland monitoring

A preserve needs to map invasive vegetation and open water.

| Question | Team response |
| --- | --- |
| Sensor and why |  |
| Fit-for-purpose resolution |  |
| Useful remote product |  |
| Ground data |  |
| Important limitation |  |

## Coastal erosion

A manager needs post-storm shoreline and dune-change evidence.

| Question | Team response |
| --- | --- |
| Sensor and why |  |
| Fit-for-purpose resolution |  |
| Useful remote product |  |
| Ground data |  |
| Important limitation |  |

## Forest management

A forester needs canopy-gap and storm-damage information.

| Question | Team response |
| --- | --- |
| Sensor and why |  |
| Fit-for-purpose resolution |  |
| Useful remote product |  |
| Ground data |  |
| Important limitation |  |

# Day 8: Accuracy, Ethics, Safety, and Regulation

Essential question When is a technically possible drone investigation scientifically and socially responsible?

Name: ______________________________    Date: __________________

## Activity 8A - Error and responsibility audit

| Scenario | Scientific / safety / ethical issue | Responsible response |
| --- | --- | --- |
| Deep shadows cross half the vegetation plot |  |  |
| Nesting birds flush when the aircraft approaches |  |  |
| Imagery includes neighboring backyards |  |  |
| Coordinates are shifted several meters |  |  |
| A team wants to fly near controlled airspace |  |  |

## Claim-Evidence-Reasoning

Prompt: Drone remote sensing should replace most ground-based environmental monitoring.

Claim

________________________________________________________________________________

________________________________________________________________________________

Evidence

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Reasoning and qualification

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

# Day 9: Capstone Investigation: Analyze the Evidence

Essential question Which part of Marshview Wetland should be investigated first, and why?

Name: ______________________________    Date: __________________

## Client brief

Marshview Wetland Preserve staff observed patches of discoloration in emergent vegetation. Your consulting team must identify the zone that warrants priority ground investigation. Use the simulated natural-color orthomosaic, NDVI surface, and sample table from a DJI Mavic 3 Multispectral demonstration mission flown at 200 ft AGL (61 m). State what the evidence does not establish.

## Sample observations

| ID | Zone | Red | NIR | NDVI | Depth cm | Field note |
| --- | --- | --- | --- | --- | --- | --- |
| S1 | A | 0.18 | 0.58 | 0.5 | 22 | Dense emergent vegetation |
| S2 | A | 0.2 | 0.62 | 0.51 | 18 | Dense emergent vegetation |
| S3 | A | 0.17 | 0.55 | 0.53 | 25 | No visible discoloration |
| S4 | B | 0.25 | 0.51 | 0.34 | 9 | Patchy vegetation |
| S5 | B | 0.29 | 0.49 | 0.26 | 7 | Yellow-brown leaves |
| S6 | B | 0.31 | 0.46 | 0.19 | 5 | Bare soil between stems |
| S7 | C | 0.14 | 0.08 | -0.27 | 61 | Open water |
| S8 | C | 0.12 | 0.1 | -0.09 | 54 | Floating vegetation sparse |
| S9 | C | 0.15 | 0.13 | -0.07 | 48 | Open water edge |
| S10 | D | 0.21 | 0.6 | 0.48 | 15 | Mixed wetland vegetation |
| S11 | D | 0.23 | 0.57 | 0.43 | 13 | Mixed wetland vegetation |
| S12 | D | 0.2 | 0.55 | 0.47 | 17 | No visible discoloration |

## Investigation organizer

Priority zone and claim

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Evidence 1: quantitative

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Evidence 2: spatial or qualitative

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Alternative explanation

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Limitation or uncertainty

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Additional data and sampling plan

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Proportional management recommendation

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

# Day 10: Scientific Briefing and Individual Assessment

Essential question How should uncertainty shape an environmental recommendation?

Name: ______________________________    Date: __________________

## Briefing outline

Environmental problem and research question

Data and sensor logic

Priority zone

At least two pieces of evidence, including one quantitative

Interpretation and alternative explanation

Limitations and uncertainty

Additional data

Management recommendation

Peer feedback - strongest evidence

________________________________________________________________________________

________________________________________________________________________________

Peer feedback - question or challenge

________________________________________________________________________________

________________________________________________________________________________

Revision I will make

________________________________________________________________________________

________________________________________________________________________________

Final reflection: How did uncertainty change your recommendation?

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

## Capstone submission checklist

I separated direct observations from interpretations.

I included at least one quantitative observation.

I explained why the priority zone matters.

I addressed water as a reason for low NDVI in Zone C.

I named uncertainty and additional data.

My recommendation is proportional to the evidence.

Maps/figures have titles and are referenced in my explanation.