INSTRUCTOR-ONLY RESOURCE

# Introduction to Drone Remote Sensing for Environmental Science

Formative Assessment Guide, Summative Assessment, Answer Keys, and Rubric

| AUDIENCE AP Environmental Science and introductory undergraduate environmental science | LENGTH 10 instructional days | RESOURCE Assessment and scoring |
| --- | --- | --- |

Driving question: How can drone-based remote sensing help environmental scientists observe, measure, and manage environmental change?

# Assessment architecture

| Component | Recommended weight | Evidence |
| --- | --- | --- |
| Capstone assessment | 70% | Team or individual report / briefing scored with analytic rubric |
| Individual concept assessment | 30% | Independent selected-response and constructed-response items |

Use daily checks diagnostically. Do not grade early misconceptions as final mastery. Accept scientifically equivalent wording when the evidence and reasoning are correct.

# Daily formative answer guide

| Day | Look-for / sample response |
| --- | --- |
| 1 | A suitable response names a phenomenon such as shoreline change or patch-scale vegetation stress, explains the value of broad spatial coverage/pattern recognition, and states a needed field check. |
| 2 | Question -> sensor/plan -> collection -> processing -> GIS/quantitative analysis -> interpretation -> decision. Quality checks may include calibration, overlap, airspace/site check, positional control, and ground validation. |
| 3 | Thermal sensor; it records emitted infrared energy/apparent surface-temperature patterns. A warm anomaly may indicate discharge, but confirmation requires temperature measurements and context. |
| 4 | A 30 m pixel can summarize patterns across many hectares but mixes individual crowns and surroundings. Individual-tree diagnosis requires much finer detail and validation. |
| 5 | NDVI = (0.60 - 0.20)/(0.60 + 0.20) = 0.50. Defensible: strong positive vegetation signal relative to lower-value areas. Overreach: proves a healthy native ecosystem. |
| 6 | A DSM represents upper surfaces such as canopies and roofs. A DTM aims to represent bare earth, which requires filtering/classification and can be difficult under dense cover. |
| 7 | Answers vary. Strong responses align sensor/resolution to the question and add a ground observation that could reduce uncertainty. |
| 8 | Preferred position: remote and ground methods are complementary. Remote sensing adds coverage and pattern detection; field work provides direct measurements, causes, calibration, and validation. |
| 9 | Zone B is the priority because of consistently lower positive NDVI and stress-related field notes. Zone C's negative values are associated with open water. Causes remain unconfirmed. |
| 10 | Strong peer feedback checks evidence, alternative explanations, uncertainty, and whether the recommendation is proportional to the evidence. |

# Individual concept assessment - 30 points

Suggested time: 25-35 minutes. Calculator permitted. Complete independently.

1. Which statement best defines remote sensing?

A. Directly collecting soil in the field

B. Obtaining information without physical contact by recording energy or range

C. Drawing a map from memory

D. Flying any camera above ground

2. Which sensor is the best starting choice for mapping an anomalously warm discharge plume?

A. Thermal

B. RGB

C. Lidar

D. Magnetometer

3. A multispectral camera used for NDVI must include which two measurements?

A. Blue and thermal

B. Green and lidar

C. Red and near-infrared

D. Elevation and temperature

4. Which resolution describes pixel ground dimensions?

A. Spectral

B. Spatial

C. Temporal

D. Radiometric

5. Why can the finest spatial resolution be a poor operational choice?

A. Fine pixels contain no information

B. Fine resolution may increase data volume, time, cost, and privacy risk

C. Satellites never have fine pixels

D. Fine resolution prevents georeferencing

6. What is the best interpretation of high NDVI?

A. Proof of high biodiversity

B. Proof no invasive species are present

C. A relatively strong vegetation reflectance signal that needs context

D. Direct measurement of photosynthetic rate

7. What product is a corrected, georeferenced image mosaic?

A. Orthomosaic

B. Flight log

C. Raw photograph

D. DTM

8. A digital surface model commonly includes:

A. Only bare earth

B. Canopies and buildings

C. Only water depth

D. Only latitude and longitude

9. What is the strongest use of ground observations?

A. Decorate a map

B. Replace all image processing

C. Validate and interpret remotely sensed patterns

D. Increase flight altitude

10. Which statement is scientifically strongest?

A. Imagery is objective and error-free

B. A visible pattern proves its cause

C. Remote sensing and field sampling often provide complementary evidence

D. Legal flight guarantees ethical practice

11. In the Marshview dataset, why is Zone C not the priority vegetation-stress zone?

A. It has no samples

B. Negative NDVI is associated with mapped open water

C. It has the highest NDVI

D. It is outside the study area

12. Before an optional real flight, which action is appropriate?

A. Assume school property is unrestricted

B. Check current FAA rules, airspace, permission, safety, privacy, and wildlife constraints

C. Fly first and document later

D. Use this worksheet as flight authorization

## Constructed response

13. Calculate NDVI for Red = 0.24 and NIR = 0.56. Show work and interpret cautiously. (3 points)

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

14. Explain the difference between what a multispectral sensor measures and what a scientist may infer about vegetation. (3 points)

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

15. A team maps individual diseased trees using 30 m pixels. Explain the scale problem and one improvement. (3 points)

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

16. Name two photogrammetry quality factors and explain how each affects the output. (3 points)

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

17. Give one privacy or wildlife concern and one practical mitigation. (2 points)

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

18. Marshview Zone B has NDVI values 0.19-0.34 and patchy/discolored vegetation notes. Write a claim, cite two pieces of evidence, state one uncertainty, and recommend one next step. (4 points)

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

# Individual assessment answer key

Selected response (1 point each): 1-B, 2-A, 3-C, 4-B, 5-B, 6-C, 7-A, 8-B, 9-C, 10-C, 11-B, 12-B.

| Item | Scoring guidance |
| --- | --- |
| 13 | NDVI = (0.56 - 0.24)/(0.56 + 0.24) = 0.32/0.80 = 0.40 (2); interpretation identifies a positive vegetation signal but avoids diagnosis (1). |
| 14 | Sensor records reflected energy in bands (1); scientist infers vegetation condition/pattern (1); interpretation requires context/validation (1). |
| 15 | 30 m pixels mix multiple trees/background and cannot reliably resolve individuals (2); use finer imagery and field validation (1). |
| 16 | One point each for two valid factor-effect pairs, e.g., overlap -> feature matching; control/RTK -> positional accuracy; motion/lighting -> blur or seam/calibration error. Maximum 3. |
| 17 | Valid concern (1) and feasible mitigation (1), e.g., nesting disturbance -> buffers/timing/stop criteria; backyard imagery -> minimize collection/access/retention. |
| 18 | Zone B claim (1); two relevant pieces of evidence (1); uncertainty/alternative cause (1); targeted ground sampling or proportional next step (1). |

# Capstone assignment

Role: environmental consulting team. Client: Marshview Wetland Preserve. Task: identify the zone needing priority investigation and recommend an evidence-proportionate response.

## Required product

4-6 page scientific assessment, or

8-10 slide briefing with equivalent speaker notes, or

5-7 minute oral/narrated briefing with an accessible evidence handout.

## Required elements

Environmental problem and focused research question

Sensor/data explanation

Map and table analysis

At least one quantitative observation

Environmental interpretation

Alternative explanation

Limitations and uncertainty

Additional data and sampling plan

Management recommendation

Clear source and figure attribution

# 100-point analytic rubric

| Criterion | Pts | Exemplary | Proficient | Developing | Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem and question | 10 | Precise, decision-relevant question | Clear question with minor gap | Broad or partly measurable | Missing or inaccurate |
| Sensor/data logic | 15 | Correctly links measurement, product, and inference | Mostly correct with minor omission | Names tools but weak logic | Major errors or unsupported choice |
| Data analysis | 20 | Integrates map, table, pattern, and calculations accurately | Uses relevant evidence with minor omission | Partial evidence or calculation errors | Minimal or incorrect analysis |
| Environmental interpretation | 15 | Calibrated inference with plausible alternatives | Sound interpretation with some qualification | Overgeneralized or weakly connected | Unsupported causal claim |
| Quantitative support | 10 | Accurate, meaningful quantitative evidence | Accurate value with basic explanation | Value present but weakly interpreted | Absent or inaccurate |
| Limitations/uncertainty | 10 | Specific limitations tied to confidence and decisions | Relevant limitations identified | Generic limitation list | Uncertainty ignored |
| Additional data | 5 | Targeted sampling would discriminate among explanations | Useful follow-up data | Vague follow-up | No relevant follow-up |
| Recommendation | 10 | Feasible and proportional; criteria/trade-offs clear | Evidence-supported next step | Partly supported or too broad | Unsupported or unsafe |
| Communication | 5 | Clear, accessible, well-labeled visuals and sources | Generally clear | Organization or labeling problems | Difficult to follow |

## Performance bands

90-100: Exemplary

80-89: Proficient

70-79: Developing

Below 70: Beginning / revision needed

# Capstone model response

Priority claim: Zone B should receive the first targeted ground investigation. Samples S4-S6 show lower positive NDVI (0.19-0.34) than the densely vegetated Zone A (0.50-0.53) and mixed-vegetation Zone D (0.43-0.48). Field notes in Zone B independently report patchiness, yellow-brown leaves, and exposed soil. Zone C has lower or negative NDVI, but the map and notes identify open water, so it is not the strongest evidence of stressed emergent vegetation. The packet does not establish the cause of Zone B's pattern; water depth, species composition, phenology, soil/salinity, pest/pathogen indicators, recent disturbance, calibration, and repeat-date imagery would help test alternatives. A proportional recommendation is a stratified ground survey of Zone B and reference plots in Zones A and D before treatment or restoration decisions.

# Sources and currency note

Standards and flight rules can change. The references below were checked on September 1, 2026. Instructors should recheck FAA and local requirements before any real-world flight. This unit does not require students to operate a drone.

College Board, AP Environmental Science

Next Generation Science Standards, HS-ESS3-4

Next Generation Science Standards, HS-ESS3-6

Next Generation Science Standards, HS-LS2-7

FAA, Recreational Flyers and Small UAS overview

USGS, Landsat Data Access

OpenAerialMap